Treedix USB-C Vertical Female Breakout Board — TRX-USBC-VBK Rev 1.0 ¦ June 2026 ¦ UNOFFICIAL / INFERRED

Inferred Specification Document · Breakout Module Treedix USB-C Vertical Female Breakout Board 10-Pin Breakout Module — Inferred Specification Document Part Number: TRX5-0698 (confirmed — Amazon Item Details) Revision: 1.0 Document Type: Inferred / Community Specification Date: June 2026 Status: ⚠ UNOFFICIAL — INFERRED Pages: 1 (single-page HTML) Standards Ref: USB Type-C Spec Rev 2.1 (USB-IF), USB PD Rev 3.1 Connector: USB Type-C Female, Vertical, 10-Pin Breakout

⚠ IMPORTANT DISCLAIMER — UNOFFICIAL INFERRED SPECIFICATION This document is an inferred specification assembled from publicly available product listing data, community measurements, and the USB Type-C specification (USB-IF). It is NOT an official datasheet issued by Treedix. No official datasheet has been published by the manufacturer at the time of this writing. All values are best estimates and should be independently verified before use in production designs. Treedix and the author bear no liability for errors or omissions. This document is intended for prototyping and hobby use reference only.

# 1 Overview

The Treedix USB-C Vertical Female Breakout Board is a small form-factor PCB module that exposes the key signal and power pins of a USB Type-C receptacle on a 2.54 mm (0.1 inch) pitch through-hole header. It is designed for prototyping, breadboard integration, and DIY electronics projects where a mechanically robust, easily wired USB-C port is required.

The vertical mounting orientation of the USB-C socket allows the connected cable to exit perpendicular to the host PCB plane, facilitating panel-mount installations in enclosures and project boxes. Ten pins are broken out from the USB-C receptacle, covering power (VBUS, GND), USB 2.0 data (D1+, D1−, D2+, D2−), configuration channel (CC1, CC2), and sideband use (SBU1, SBU2). High-speed SuperSpeed (USB 3.x) lanes are not broken out on this variant.

### Intended Applications

- Breadboard prototyping and Dupont wire interfacing

- Panel-mount USB-C port replacement in enclosures

- USB 2.0 data path testing and development

- DIY charging circuits (5V supply input)

- Configuration channel (CC) experiments and USB Power Delivery research

- Maker and hobbyist electronics projects

Module at a Glance: USB Type-C Female Vertical ¦ 10-Pin, 2.54 mm Pitch ¦ Passive (No Active Components) ¦ FR4 PCB ¦ ENIG Surface Finish ¦ USB 2.0 Hi-Speed (480 Mbps max)

Figure 1-1: Treedix USB-C Vertical Female Breakout Board (2-pack). Photo sourced from Amazon product listing.

# 2 Features

- Vertical USB Type-C female receptacle — reversible insertion eliminates incorrect plug-in errors

- 10-pin, 2.54 mm (0.1″) pitch through-hole breakout header — breadboard and Dupont wire compatible

- Exposes all key USB-C signals: GND, VBUS, CC1, CC2, D1+, D1−, D2+, D2−, SBU1, SBU2

- Positioning / mounting holes on PCB for M2/M2.5 mechanical fixation

- FR4 fiberglass PCB substrate, 1.6 mm nominal thickness

- Gold-plated (ENIG) through-hole pads for long-term corrosion resistance

- Pin labels silk-screened on PCB top and bottom layers for easy identification

- Compact and lightweight module — approximately 2 g per board

- Compatible with both straight and right-angle 0.1″ pin headers (included in package)

- No active components — purely passive interconnect; no ICs, no ESD diodes

- Available in 2-piece packs; inferred ASIN: B09B1P6VJQ

- Compatible with standard USB Type-C cables (USB 2.0 data operation)

# 3 Mechanical Drawing

Figure 3-1: Top-view block diagram (ASCII art). Not to scale. All dimensions inferred / estimated.

┌──────────────────────────────────────────────────┐ │ USB-C RECEPTACLE BODY │ │ ┌────────────────────────────────┐ │ │ │ ▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓ │ │ │ │ ▓ USB Type-C • Female ▓ │ │ │ │ ▓ Vertical Mount (10-pin) ▓ │ │ │ │ ▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓▓ │ │ │ └────────────────────────────────┘ │ │ ◎ MTG MTG ◎ │ │ │ │ ○ ○ ○ ○ ○ │ │ [1] [2] [3] [4] [5] │ │ GND VBUS CC1 CC2 D1+ │ │ │ │ ○ ○ ○ ○ ○ │ │ [6] [7] [8] [9] [10] │ │ D1- D2+ D2- SBU1 SBU2 │ │ │ │ ◎ MTG MTG ◎ │ └──────────────────────────────────────────────────┘ ←────────────── ~20 mm ──────────────→ (estimated) ↕ ~15 mm (estimated) Note: ◎ = Mounting hole (est. Ø2.5 mm) ○ = Through-hole breakout pin (2.54 mm pitch)

## 3.1 Mechanical Dimensions (Inferred)

Note: All mechanical dimensions are inferred or estimated from product images and community measurements. No official mechanical drawing has been published by Treedix. Verify critical dimensions with calipers before designing enclosures or adapters.

| Parameter | Value (Inferred) | Tolerance | Notes |
| --- | --- | --- | --- |
| PCB Length | 20 mm | ±0.5 mm | Confirmed — Amazon (ASIN B0D31GG6WD) & Walmart product listing |
| PCB Width | 15 mm | ±0.5 mm | Confirmed — Amazon (ASIN B0D31GG6WD) & Walmart product listing |
| PCB Thickness | 1.6 mm | ±0.1 mm | Standard FR4 nominal thickness |
| Total Height (with connector) | ~10–12 mm | ±1 mm | Includes USB-C socket body above PCB |
| Pin Pitch | 2.54 mm | ±0.05 mm | Standard 0.1″ header pitch |
| Pin Row Spacing | 2.54 mm | ±0.05 mm | Single row of 10 pins |
| Mounting Hole Diameter | ~2.5 mm | ±0.2 mm | Estimated from product images; M2 or M2.5 |
| Module Mass | ~10 g | ±1 g | Per Amazon/Walmart listing: 0.02 kg listed for the 2-pack = ~10 g per board. Prior estimate of ~2 g was incorrect. |
| PCB Material | FR4 | — | Fiberglass epoxy laminate |
| Copper Weight | 1 oz/ft² (estimated) | — | Standard PCB copper weight; unconfirmed |
| Surface Finish | ENIG (Gold) | — | Per Treedix product description |

# 4 Pin Descriptions

The following table defines all 10 breakout pins exposed by the Treedix USB-C Vertical Breakout Board. Pin numbers (1–10) are assigned in this document for reference. Verify against the PCB silk-screen marking on your specific board revision.

| Pin # | Label | USB-C Source Pins | Direction | Description | Typical Use |
| --- | --- | --- | --- | --- | --- |
| 1 | GND | A1, B12, A12, B1 | — | Power/signal ground return. All USB-C ground pins are shorted together on this breakout. | Connect to system ground |
| 2 | VBUS | A4, A9, B4, B9 | Input / Output | USB bus power rail. Carries 5V by default; higher voltages possible with USB PD negotiation. All VBUS pins are shorted together on the module. | Power input from host / power output to device |
| 3 | CC1 | A5 | Bidirectional | Configuration Channel 1. Used for plug orientation detection, current advertisement, and USB Power Delivery messaging. | Pull-down 5.1 kΩ to GND for UFP/sink role; leave open if source |
| 4 | CC2 | B5 | Bidirectional | Configuration Channel 2. Mirrors CC1 for the flipped connector orientation. Only one CC pin will be active per insertion orientation. | Same as CC1; in cable the other CC becomes VCONN |
| 5 | D1+ | A6 | Bidirectional | USB 2.0 Data+ (Side A). Half of the primary USB 2.0 differential pair. Carries high-speed data up to 480 Mbps. | Connect to D+ of downstream USB 2.0 device or host |
| 6 | D1− | A7 | Bidirectional | USB 2.0 Data− (Side A). Complementary signal to D1+. Forms the differential pair. | Connect to D− of downstream USB 2.0 device or host |
| 7 | D2+ | B7 | Bidirectional | USB 2.0 Data+ (Side B). Duplicate pair for reversed connector orientation. D1+ and D2+ may be shorted together on-board; verify with DMM. | Same as D1+; or leave open / short to Pin 5 |
| 8 | D2− | B6 | Bidirectional | USB 2.0 Data− (Side B). Duplicate pair for reversed orientation. Complementary to D2+. | Same as D1−; or leave open / short to Pin 6 |
| 9 | SBU1 | A8 | Bidirectional | Sideband Use 1. Low-speed sideband channel. Used for DisplayPort AUX channel, audio adapter accessory mode, and other alternate modes. | Leave unconnected if not using alternate modes |
| 10 | SBU2 | B8 | Bidirectional | Sideband Use 2. Complementary sideband channel for reversed orientation. Mirrors SBU1 for the flipped plug. | Leave unconnected if not using alternate modes |

Pin Numbering Note: The pin numbering (1–10) above is assigned in this document for reference clarity. The actual PCB may use a different silk-screen numbering convention or sequence. Always verify against your board's physical markings before wiring.

# 5 Electrical Characteristics

All values derived from the USB Type-C Specification Rev 2.1 (USB-IF) and apply to standard USB 2.0 operation. No independent electrical testing of this specific module has been performed. PCB-specific parameters (trace resistance, capacitance, current capacity) are uncharacterized.

## 5.1 Absolute Maximum Ratings

Stresses exceeding these values may permanently damage the device. These are stress ratings only, not functional operating conditions.

| Parameter | Symbol | Min | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- |
| VBUS Voltage | VVBUS | −0.5 | 5.5 | V | Standard USB 5V operation |
| VBUS Voltage (with PD) | VVBUS_PD | — | 20 | V | Requires PD negotiation; PCB trace ratings unknown — use with caution |
| VBUS Continuous Current (est.) | IVBUS | — | 3 | A | Inferred; PCB trace width unknown — derate for safety |
| CC Pin Voltage | VCC | −0.3 | 5.5 | V | USB-IF specification limit |
| D+/D− Pin Voltage | VD | −0.3 | 3.6 | V | USB 2.0 data signal levels |
| SBU Pin Voltage | VSBU | −0.3 | 3.6 | V | Low-speed sideband signal |
| Operating Temperature | TOP | −20 | +85 | °C | Inferred from FR4 glass transition temperature |
| Storage Temperature | TST | −40 | +125 | °C | Per FR4 substrate standard rating |

## 5.2 Recommended Operating Conditions

| Parameter | Symbol | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| VBUS Supply Voltage | VVBUS | 4.75 | 5.00 | 5.25 | V |
| VBUS Current (USB 2.0 nominal) | IVBUS | — | 500 | 900 | mA |
| VBUS Current (USB-C 1.5A mode) | IVBUS_1.5 | — | 1500 | 1500 | mA |
| VBUS Current (USB-C 3.0A mode) | IVBUS_3.0 | — | 3000 | 3000 | mA |
| D+/D− Differential Impedance | ZD | 80 | 90 | 100 | Ω |
| CC Pull-down Resistance (Rd, sink) | Rd | 4.59 | 5.10 | 5.61 | kΩ |
| CC Pull-up Resistance (Rp, default source) | Rp_def | 47.7 | 56 | 63.8 | kΩ |
| CC Pull-up Resistance (Rp, 1.5A) | Rp_1.5 | 20 | 22 | 24 | kΩ |
| CC Pull-up Resistance (Rp, 3.0A) | Rp_3.0 | 9 | 10 | 11 | kΩ |

## 5.3 USB 2.0 Data Signaling

| Parameter | Value | Unit |
| --- | --- | --- |
| Maximum Data Rate | 480 | Mbps (Hi-Speed USB 2.0) |
| D+/D− Signal Swing (HS mode) | 400 | mVdiff |
| D+/D− Common Mode Voltage (HS) | 0.2 | V |
| D+ Pull-up (Full Speed detection) | 1.5 | kΩ to 3.3V (external, on device side) |

D2+/D2− Note: The D2+ / D2− pins are the USB-C B-side duplicates of D1+ / D1−. On many simple breakout boards, these are shorted to D1+ / D1− through the connector's internal mirroring. Measure continuity with a DMM before assuming independent routing on your specific board.

## 5.4 USB Power Delivery — Informational

The Treedix 10-pin breakout exposes CC1 and CC2 pins, enabling USB Power Delivery (USB PD) experimentation. However, the board contains NO PD controller IC. External circuitry is required for PD negotiation above default 5V/0.9A. Operating at elevated PD voltages (9V, 12V, 15V, 20V) requires verifying PCB trace current capacity, which has not been published by Treedix.

| PD Profile | Voltage | Max Current | Max Power | Board Status |
| --- | --- | --- | --- | --- |
| Default (USB 2.0) | 5V | 500–900 mA | 2.5–4.5 W | ✔ Safe |
| USB-C 1.5A | 5V | 1.5 A | 7.5 W | ✔ Safe (verify traces) |
| USB-C 3.0A | 5V | 3.0 A | 15 W | ⚠ Verify PCB traces |
| PD Profile 1 | 5V | up to 3A | 15 W | ⚠ Verify PCB traces |
| PD Profile 2 | 9V | up to 3A | 27 W | ✘ Not recommended |
| PD Profile 3 | 15V | up to 3A | 45 W | ✘ Not recommended |
| PD Profile 4 | 20V | up to 5A | 100 W | ✘ Dangerous — do not use |

⚠ WARNING: Using PD voltages above 5V without knowing the PCB trace current capacity and insulation clearances may cause board damage, fire, or injury. Do not exceed 5V / 3A on this module until trace ratings are independently confirmed by measurement.

# 6 Usage Notes

## 6.1 Basic Wiring — USB 2.0 Data + Power

For a basic USB 2.0 device connection to a microcontroller or USB controller IC:

- Connect Pin 1 (GND) to system ground

- Connect Pin 2 (VBUS) to 5V supply rail

- Connect Pin 5 (D1+) to downstream D+

- Connect Pin 6 (D1−) to downstream D−

- Add 5.1 kΩ resistors from CC1 (Pin 3) and CC2 (Pin 4) to GND to identify this board as a USB-C sink / UFP (Upstream Facing Port)

## 6.2 Basic Wiring — 5V Power Input Only (Charger Port)

For a 5V charging-only input port (no data transfer):

- Connect Pin 1 (GND) to ground

- Connect Pin 2 (VBUS) to your 5V load or LDO input

- Add 5.1 kΩ pull-downs on CC1 and CC2 (Pins 3 and 4) to GND — this enables standard USB-C current advertisement recognition by the host charger

- Leave Pins 5–10 (D1+, D1−, D2+, D2−, SBU1, SBU2) unconnected

## 6.3 Pin Header Options

The board ships with both straight and right-angle 0.1″ pin headers. Choose based on your mounting orientation:

- Straight header: PCB stands vertically above a breadboard or parent PCB; USB-C port faces upward

- Right-angle header: PCB lies flat with USB-C port facing outward — ideal for side panel mounting

## 6.4 Mechanical Mounting

Mounting holes (estimated Ø2.5 mm) are located at the board corners for M2 or M2.5 hardware. Verify exact hole diameter with calipers before ordering standoffs, as the true diameter has not been officially published.

## 6.5 SuperSpeed / USB 3.x — Not Supported

The Treedix 10-pin breakout does NOT expose the SuperSpeed TX/RX differential pairs (USB-C pins A2, A3, B10, B11, A10, A11, B2, B3). Maximum data rate is limited to USB 2.0 Hi-Speed (480 Mbps). Use a 24-pin full breakout board for USB 3.x SuperSpeed applications.

## 6.6 ESD and Overvoltage Protection

No ESD protection diodes or transient voltage suppressors (TVS) are present on the PCB. This is a purely passive breakout. For applications exposed to electrostatic discharge or transient voltages, add external ESD protection (e.g., PRTR5V0U2X, USBLC6-2, or equivalent) on the D+/D− lines and VBUS before connecting to sensitive ICs.

# 7 Schematic Overview

The board contains no active components. All routing is direct copper trace from USB-C receptacle pad to through-hole breakout pin. The functional topology is shown below:

USB-C RECEPTACLE (Vertical, Female) ───────────────────────────────────────────────────────── A1 / B12 / A12 / B1 (GND) ──────────────→ Pin 1 │GND │ A4 / A9 / B4 / B9 (VBUS) ──────────────→ Pin 2 │VBUS│ A5 (CC1) ──────────────→ Pin 3 │CC1 │ B5 (CC2) ──────────────→ Pin 4 │CC2 │ A6 (D1+) ──────────────→ Pin 5 │D1+ │ A7 (D1-) ──────────────→ Pin 6 │D1- │ B7 (D2+) ──────────────→ Pin 7 │D2+ │ B6 (D2-) ──────────────→ Pin 8 │D2- │ A8 (SBU1) ──────────────→ Pin 9 │SBU1│ B8 (SBU2) ──────────────→ Pin 10 │SBU2│ ───────────────────────────────────────────────────────── SuperSpeed TX/RX pairs (A2, A3, B10, B11, A10, A11, B2, B3): NOT BROKEN OUT — internally unused ───────────────────────────────────────────────────────── No resistors, capacitors, or active components on-board. Direct trace routing only. Passive breakout.

# 8 Typical Application Circuits

## 8.1 UFP (Upstream Facing Port) — USB 2.0 Device

Use this configuration when connecting the breakout board to a microcontroller or USB device controller IC:

| Signal | Breakout Pin | Connect To | External Component Required |
| --- | --- | --- | --- |
| GND | Pin 1 | System GND | — |
| VBUS | Pin 2 | Device 5V input / LDO | 10 µF bulk capacitor recommended |
| CC1 | Pin 3 | GND | 5.1 kΩ pull-down resistor (required for sink role) |
| CC2 | Pin 4 | GND | 5.1 kΩ pull-down resistor (required for sink role) |
| D1+ | Pin 5 | MCU / USB controller D+ | — |
| D1− | Pin 6 | MCU / USB controller D− | — |
| D2+ | Pin 7 | Leave open or short to Pin 5 | — |
| D2− | Pin 8 | Leave open or short to Pin 6 | — |
| SBU1 | Pin 9 | Leave open (no connection) | — |
| SBU2 | Pin 10 | Leave open (no connection) | — |

## 8.2 Simple 5V Power Input (Sink / Charger Port)

Use this configuration for a power-only USB-C input port with no data lines:

| Signal | Breakout Pin | Connect To | External Component Required |
| --- | --- | --- | --- |
| GND | Pin 1 | System GND | — |
| VBUS | Pin 2 | 5V Load / LDO / Battery Charger IC | TVS diode + polyfuse recommended on VBUS |
| CC1 | Pin 3 | GND | 5.1 kΩ resistor (enables current advertisement) |
| CC2 | Pin 4 | GND | 5.1 kΩ resistor |
| D1+, D1−, D2+, D2−, SBU1, SBU2 | Pins 5–10 | Leave open (no connection) | — |

CC Pull-down Requirement: Without 5.1 kΩ pull-downs on both CC pins, a USB-C charger will not recognize the device as a valid sink and may refuse to supply power or limit current to below 100 mA. This is per the USB Type-C specification (Rd = 5.1 kΩ).

# 9 Ordering Information

| Description | Source | Identifier | Notes |
| --- | --- | --- | --- |
| Treedix USB-C Vertical Breakout (2-pack) | Amazon.com | B0D31GG6WD (confirmed ASIN) ¦ UPC: 644197776388 | Sold by Treedix Store; Amazon BSR: #3,193 in Audio & Video Connectors & Adapters (as of June 2026) |
| Treedix USB-C Vertical Breakout | Treedix Official Website | TRX5-0698 (confirmed — Amazon Item Details) | treedix.com; model number confirmed from Amazon Item Details panel |
| Treedix USB-C 24-Pin Full Breakout (separate product) | Treedix Official | TRX5-0248 | Different product — all 24 pins broken out; not the vertical 10-pin variant covered by this document |
| 3D CAD Model (STEP / Fusion 360) | GrabCAD | "Treedix USB-C Breakout Board" | Community-created by Daniel Nering, May 18, 2022. STEP + F3D files; 1,664 downloads, 35 likes. Creator note: "for downstream devices, not host devices." Not official. |
| PCB Schematic / Footprint | EasyEDA | "TREEDIX USB-C Breakout PCB" | Community-created; verify accuracy before use in PCB layout |

# 10 Disclaimers and Limitations

- No Official Datasheet: As of June 2026, Treedix has not published an official datasheet for this module. All values in this document are estimated or derived from indirect sources and community measurements.

- PCB Trace Ratings Unknown: Maximum current-carrying capacity of the PCB traces has not been independently verified. The 3A figure referenced in Amazon listings refers to the USB-C connector specification, not confirmed PCB trace capacity.

- High-Voltage PD Risk: Do not operate VBUS above 5.5V without verifying PCB trace ratings, component clearances, and creepage distances. Doing so may result in board damage, fire, or injury.

- No Active Protection: No ESD protection, overvoltage protection, overcurrent limiting, or reverse-polarity protection is present on the PCB. The user is responsible for adding appropriate external protection.

- SuperSpeed Not Supported: USB 3.x SuperSpeed differential pairs are not exposed on this 10-pin breakout. Maximum data rate is 480 Mbps (USB 2.0 Hi-Speed).

- Not for Safety-Critical Applications: This board is intended for prototyping and hobby use only. It is not suitable for medical, industrial safety, automotive, aviation, or other safety-critical applications.

- D+/D− Shorting Behavior: On some USB-C receptacle variants, D+ and D− from both connector sides are internally shorted to a single node within the connector. The routing of D2+/D2− relative to D1+/D1− should be verified by the user for their specific board revision using a digital multimeter.

- Treedix Trademark Notice: Treedix is a trademark of Treedix Technology. This document is independently authored and is not endorsed by, affiliated with, or approved by Treedix. All trademarks are the property of their respective owners.

# 11 Related Resources

| Resource | Description |
| --- | --- |
| USB Type-C Cable and Connector Specification Rev 2.1 | USB Implementers Forum (USB-IF) — defines all pin functions, electrical requirements, and cable assembly specifications |
| USB Power Delivery Specification Rev 3.1 | USB-IF — defines PD negotiation protocol over CC pins; mandatory reference for PD implementation |
| IEC 62680-1-3 | International standard for USB Type-C connectors (equivalent to USB-IF Type-C specification) |
| GrabCAD Model by Daniel Nering (May 18, 2022) | Community 3D STEP / Fusion 360 F3D model of this specific Treedix board (TRX5-0698); 1,664 downloads, 35 likes. Useful for enclosure design. Creator notes: intended for downstream (UFP/device) use only, not host (DFP) use. |
| EasyEDA — TREEDIX USB-C Breakout PCB | Community PCB schematic and component footprint; verify accuracy before using in production PCB layout |
| Amazon ASIN B0D31GG6WD | Confirmed product listing (ASIN B0D31GG6WD, UPC 644197776388); model number TRX5-0698 confirmed from Item Details panel. Customer photos, dimensions, and Q&A are primary public sources of specifications. BSR: #3,193 in Audio & Video Connectors & Adapters (June 2026). |
| PRTR5V0U2X ESD Protection IC | NXP Semiconductors — recommended external ESD protection for D+/D− lines (SOT-363 package) |
| USBLC6-2SC6 ESD Protection IC | STMicroelectronics — alternative external ESD protection for USB data and VBUS lines |

# 12 Revision History

| Rev | Date | Author / Source | Description of Changes |
| --- | --- | --- | --- |
| 0.1 | May 18, 2022 | Daniel Nering (GrabCAD Community) | First community documentation: 3D STEP/F3D CAD model of the Treedix USB-C Vertical Breakout board (TRX5-0698) created and published on GrabCAD. First public dimensional reference for the module. |
| 1.0 | June 2026 | Inferred Specification Document (Community Authored) | Initial release of this inferred datasheet. All values derived from Amazon product listing data, USB-IF specification Rev 2.1, USB PD Rev 3.1, community measurements, and GrabCAD 3D model references. No official Treedix documentation was available at time of writing. |
| 1.1 | June 2026 | Updated per live source verification | PCB dimensions confirmed 20×15 mm; mass corrected to ~10 g/board; downstream-only usage note added from GrabCAD community data. |
| 1.2 | June 8, 2026 | Live browser verification (Amazon ASIN B0D31GG6WD; GrabCAD) | Corrected ASIN from B09B1P6VJQ to B0D31GG6WD; confirmed official model number TRX5-0698 from Amazon Item Details panel; added UPC 644197776388; corrected GrabCAD upload date to May 18, 2022; noted that mounting hole diameter, total board height, and PCB copper weight remain inaccessible without login or CAD file download. |

Document prepared: June 2026. This is revision 1.0 of the community-inferred specification document for the Treedix USB-C Vertical Female Breakout Board (TRX-USBC-VBK). For corrections or community contributions, refer to the GrabCAD and EasyEDA community resources listed in Section 11.

Rev 1.0 ¦ June 2026 ¦ UNOFFICIAL — Inferred Specification TRX-USBC-VBK ¦ Treedix USB-C Vertical Female Breakout Board ¦ Not an official Treedix document