WM-D6C Servo Replacement Module — DSR-1 — System Block Diagram Two Wall Variants + Battery-Integrated Build │ STM32G0C1KCU6 │ Open Source Hardware │ CERN OHL-P v2 • MIT • CC BY 4.0

| VARIANT A USB-C Power Delivery |  | VARIANT A INPUT PATH | ► | SHARED CORE STM32G0C1KCU6 + Power Management | ► | VARIANT B INPUT PATH |  | VARIANT B Barrel Jack (Polarity-Agnostic) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| USB-C Receptacle (Panel / flex tail) VBUS D+ D− CC1 CC2 GND | ► | IP2721 PD Trigger 9V PDO Request ▼ TPS63070 Buck-Boost 9V → 6.0V regulated | ► | STM32G0C1KCU6 UFQFPN32 • 64 MHz Cortex-M0+ • 256 KB Flash • 144 KB RAM PA0 TIM2_CH1 FG Pulse Capture PA1/2/3 ADC RV601 RV602 RV603 PA4 DAC1_OUT Motor Drive Analog PA5/7 GPIO Motor EN S601 Speed Sw PA6 TIM3_CH1 PWM Out (Option B) PA8/PB15 UCPD1 CC1/CC2 PD Control PA11/12 USB D−/D+ FS 12Mbit crystal-less PA13/14 SWD PA9 USART1 Debug Digital PLL PI Servo • Fixed-point C • ISR < 500 ns USB CDC Live Tuning • DFU no ST-Link MT3608 Boost: B+1 6V → B+3 10.8V (Motor) MCP1700 LDO: B+1 6V → VDD 3.3V | ► | Polyfuse 500mA Overcurrent trip 1.5A ▼ LTC4359 Ideal Diode Bridge Polarity-agnostic • < 50mV drop ▼ SMBJ7.0A TVS + 220µF Bulk + TPS63070 Barrel jack 4.5–9V either polarity → 6.0V |  | Barrel Jack CN301 (Retained in panel) ±Polarity In 4.5V – 9V |
| CC1/CC2 → UCPD1 (PD monitoring) D+/D− → USBLC6 ESD → USB PHY VBUS → IP2721 → 9V negotiated | ▼ | ▼ 6.0V B+1 rail → WM-D6C main board ▼ 10.8V B+3 motor rail ▼ 3.3V VDD → STM32 | ▼ |  | TVS SMBJ7.0A + 220µF bulk cap 6V clean rail drives shared core |  |  |

BATTERY-INTEGRATED BUILD — POWER DAUGHTER BOARD (mounted behind battery bay) — PRIMARY / PORTABLE CONFIGURATION

| USB-C 5V (no PD) Plain 5V VBUS — 9V would over-volt charger | ► | BQ24074 Power-Path Charger DPPM: deck first, surplus to charge Supplement mode: battery assists motor surge R_ISET ≈ 1.1 kΩ → 0.8A charge (0.27C) R_ILIM ≈ 800 Ω → 2A input limit | ► | 3× 803040 LiPo (3P) 3.7V • 3000mAh • 11.1Wh Stacked 40×30×24mm in 57×32×29mm cavity ~4.6mm swell clearance • balance-match before join DW01 + FS8205 dual-FET pack protection Cells plug in → service disconnect | ► | TPS63070 Buck-Boost SYS (3.0–4.9V) → B+1 6.0V Boost mode • D ≈ 0.38 | ► | 6V → Battery terminal node → S901 power switch (native control preserved) → Machine B+1 net → CP304 harness → Module Module power-input zone UNPOPULATED Module receives B+1 (J1 pin 7 = input) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Board-to-board link: B+1 • GND • VBAT_SENSE → STM32 ADC (LiPo OCV fuel gauge) • BQ24074 CHG/PGOOD → STM32 GPIO (charge animation) • (opt) USB D+/D− Runtime: ~3–4 h per charge • charge-in-place over USB-C |  |  |  |  |  |  |  |  |

▼ WM-D6C MACHINE INTERFACE (J1 — 8-pin 1.25mm JST) ▼

| FG901 Optical Sensor FG_RAW → PA0 Level shift + BAT54 clamp | Q601 2SB733 PNP Motor DAC/PWM → Base (Option A or B) | Speed Pots RV601→PA1 RV602→PA2 RV603→PA3 ADC_IN1/2/3 | Motor Rail MT3608 B+3 10.8V → Q703/Q704 → M901 | S601 Speed Sw → PA7 Motor EN IC601 pin 7 → PA5 | B+1 Rail (6V) → Audio • Logic • Auto-Off GND (Chassis) |
| --- | --- | --- | --- | --- | --- |

| Open Source Hardware • CERN OHL-P v2 (hardware) • MIT (firmware) • CC BY 4.0 (documentation) | STM32G0C1KCU6 • UFQFPN32 • Bare-metal C • Replaces CP304 + CX20084 + CN301 |
| --- | --- |